OCP card FRU programming

Requirement:

- APT13181 main board with AMI BMC firmware.

- Linux Host running on APT13181.

- OCP Card and FRU EEPROM write protect disabled.

- Frugen tool and FRU information json file.

Step:

- Plug the OCP card to slot 0

- Power on the main board.

- When BMC ready, then power on the host server.

- Copy frugen tool and json file to host server.

- open the terminal and covert FRU info. Json file to binary file.

./frugen -j -z pnc-ob2r-g10-fru.json pnc-ob2r-g10-fru.bin

- Read FRU information:

Command for slot 0: sudo ipmitool fru print 0x01

Command for sloe 1: sudo ipmitool fru print 0x02

If OCP card exist and eeprom content is empty. The reading info is:

“unknow FRU header version 0xff”, if card is not exist the info is:

“Device not present (Unknown (0x81))

- Writing FRU information

Command: sudo ipmitool fru write 0x01 pnc-ob2r-g10-fru.bin

- Read FRU information

The ipmitool cannot recognize the OCP FRU multi-record information